# INSTANT — Energy Monitoring Website

## A Beginner’s Guide to the Components, Operation, Theory, and Code

Version: 1.0 Date: August 2026 Audience: Beginners — no prior experience with React, electronics, or cloud platforms is assumed. Subject: The INSTANT project — a web application that monitors electrical energy in real time and lets you control a relay from any browser.

# Table of Contents

- Introduction

- Before We Start — The Big Picture

- Electrical Theory You Need

- IoT and Web Concepts You Need

- System Architecture at a Glance

- Hardware Components

- The ThingSpeak Cloud Platform

- How the Website Is Operated (User Guide)

- The React Basics You Need to Read the Code

- Project Structure and File Map

- Application Startup — index.js and index.html

- The App Shell — App.js

- Global State and the Polling Engine — AppContext.js

- Talking to ThingSpeak — api/client.js

- Reusable UI Components

- The Pages

- Theming and Styling

- End-to-End Feature Walkthroughs

- Error Handling and Edge Cases

- Security Considerations

- Building, Running, and Deploying

- The Android App (APK)

- The ESP32 Firmware

- Troubleshooting

- Glossary

- Further Reading

# 1. Introduction

INSTANT is a real-time energy-monitoring web application. It shows live electrical measurements — voltage, current, power, and power factor — on a modern, animated dashboard, records history over time, and lets you switch a relay (for example, to turn a load on or off) with a single click from any browser.

The project is built from three main pieces:

- A hardware device based on the ESP32S2 microcontroller, which measures the electricity flowing through a cable using non-invasive sensors and sends the readings to the internet.

- A cloud database (ThingSpeak) that stores the readings and the settings, acting as a bridge between the device and the browser.

- A website (the “frontend”) written in React that runs entirely in the browser, reads data from the cloud, and renders it as gauges, charts, tables, and switches.

This report explains each of these pieces. It is written for someone who is comfortable using a computer but has never programmed before. Every chapter builds on the previous one, and all technical jargon is defined in the Glossary at the end.

The goals of this report are to answer four questions:

- What are the components of the system?

- How does the system operate, from sensor to screen?

- What is the theory — the electrical and web concepts behind it?

- How does the code function, file by file?

By the end, you should be able to look at the project files, follow the flow of data through the system, and explain why the website behaves the way it does.

# 2. Before We Start — The Big Picture

Imagine you want to know how much electricity a machine is using, and you also want to be able to switch that machine off from your phone, from anywhere in the world. That is exactly the problem INSTANT solves.

Here is the journey of one measurement, from the physical world to your screen:

Wall cable carrying electricity
              │
              ▼
    Sensors clamp onto the cable
    (voltage sensor + current sensor)
              │
              ▼
    ESP32 microcontroller measures the signals
              │
              ▼   (every 15 seconds)
    ESP32 sends numbers to the ThingSpeak cloud
              │
              ▼   (browser asks every 20 seconds)
    ThingSpeak returns the numbers (JSON)
              │
              ▼
    React website renders them as gauges and charts

The same cloud also carries commands in the opposite direction:

You click the relay switch on the website
              │
              ▼   (one HTTP request)
    Website writes "ON" or "OFF" to ThingSpeak
              │
              ▼   (device checks every 10 seconds)
    ESP32 reads the new value and switches the relay

Notice an important design decision: the website and the device never talk to each other directly. They both talk to ThingSpeak, which stores data and acts as a middleman. This is a common pattern called cloud-based communication, and it means the device and the website do not need to know where the other one is. The device only needs to reach the cloud; the browser only needs to reach the cloud.

Let us look at the same idea from a “roles” point of view:

| Piece | Role | Example |
| --- | --- | --- |
| Sensor layer | Measures the physical world | ZMPT101B voltage sensor, SCT-013 current sensor |
| Device layer | Reads sensors, does math, talks to the cloud | ESP32S2 running Arduino firmware |
| Cloud layer | Stores data, passes messages, handles timing | ThingSpeak channels |
| Presentation layer | Shows data, accepts user actions | React website on GitHub Pages |

The rest of this report fills in the details of each layer.

# 3. Electrical Theory You Need

You do not need an engineering degree to understand this project, but a small amount of electrical theory will make everything else clearer.

## 3.1 Voltage, Current, and Power

- Voltage (V) is the “pressure” that pushes electricity along a wire. It is measured in volts (V). In most homes the mains supply is about 220–240 V.

- Current (I) is the amount of electricity flowing. It is measured in amperes or amps (A). A phone charger might draw about 1 A; a kettle might draw about 10 A.

- Power (P) is the rate at which energy is used. It is measured in watts (W). Power is simply the product of voltage and current:

P=V×I

So a device running at 230 V drawing 2 A uses 460 W.

## 3.2 Power Factor (PF)

Real-world devices often do not use the power that they seem to use. The apparent power is V × I, but the actually useful power (the real power that heats, moves, or charges things) is smaller, because current and voltage are not perfectly in step (in phase). The ratio of real power to apparent power is called the power factor:

PF=Real PowerV×I

Power factor is a number between 0 and 1. A value close to 1 means the device uses power efficiently. Lighting and heaters have a high power factor; motors and transformers often have a lower one. INSTANT derives PF in the browser as power / (voltage × current) because the device does not send it directly.

## 3.3 Why We Measure, Not Guess

Instead of cutting the cable and inserting a meter, INSTANT uses non-invasive sensors that clamp around the outside of a wire:

- A voltage sensor measures the voltage at the terminals (in this project, a ZMPT101B which is a small voltage transformer).

- A current sensor is a clamp that measures the magnetic field around the wire (an SCT-013 “split core” current transformer). This is what makes the system safe to install — the live wire is never broken.

The microcontroller reads the two signals many times per second and calculates the RMS (root-mean-square) values of voltage and current. RMS is the standard way to describe an alternating (AC) signal: it is the equivalent steady DC value that would produce the same heating effect.

## 3.4 Safety Limits

Because electricity is dangerous, the system has safety thresholds:

- vMax — maximum allowed voltage,

- iMax — maximum allowed current,

- pMax — maximum allowed power.

If any live reading goes above its threshold, the website shows a red FAULT DETECTED banner, and the device firmware is designed to switch the relay off for safety. The thresholds themselves live in the cloud (the config channel), so you can change them from the website without touching the device.

# 4. IoT and Web Concepts You Need

INSTANT is an IoT (Internet of Things) application: physical things (the ESP32, sensors, relay) connected to the Internet. Several general web concepts are used throughout, so let us define them.

## 4.1 HTTP and REST

The HTTP protocol is how browsers and devices ask for data on the web. A client (browser or device) sends a request to a server and receives a response. Two verbs matter here:

- GET — “please give me this data” (a read).

- POST — “please store this data” (a write).

REST is a style of organizing these requests around resources (for example, “the latest reading” or “the settings”). ThingSpeak exposes a REST API: each channel has URLs you can GET or POST to.

## 4.2 JSON

JSON (JavaScript Object Notation) is a human-readable text format for structured data. Here is a JSON object representing one reading:

{
  "field1": "230.4",
  "field2": "1.52",
  "field3": "350.2",
  "created_at": "2026-08-15T10:30:00Z"
}

JSON is made of pairs of names and values, using braces {} for objects and brackets [] for lists. JavaScript (the language of the website) can turn JSON into real objects with one function call.

## 4.3 CORS (Cross-Origin Resource Sharing)

Normally, a web page at one address is not allowed to fetch data from another address — a security rule called the same-origin policy. CORS is the mechanism by which a server says “yes, I allow pages from other origins to read my data.” ThingSpeak sends Access-Control-Allow-Origin: * on all its endpoints, which means any website (or even a file on your disk) can read and write it directly. This single decision is why INSTANT needs no backend server: the browser talks straight to ThingSpeak.

## 4.4 Polling

Because the browser cannot receive data that the server decides to push, it must ask repeatedly. This is called polling: on a timer, ask “is there anything new?” In INSTANT the browser polls ThingSpeak every 20 seconds. This is simple and reliable, at the cost of a short delay (up to 20 seconds between a new reading and it appearing on screen).

## 4.5 The Single-Page Application (SPA)

INSTANT is a single-page application. Instead of loading a new HTML page for each screen (Dashboard, History, Settings), the browser loads one page once and JavaScript swaps the visible content instantly. This makes navigation feel fast and app-like. The trade-off is that JavaScript must be running — that is why the HTML contains the message “You need to enable JavaScript to run INSTANT.”

## 4.6 Static Hosting

A static website is just files (HTML, CSS, JavaScript) served as-is by a web server — there is no code running on the server. INSTANT is served by GitHub Pages, which is free static hosting. Static hosting plus CORS-open ThingSpeak is what makes the whole project cost nothing to run.

# 5. System Architecture at a Glance

The architecture is deliberately simple: device → cloud → browser, with no server of our own in the middle.

┌─────────────────────────────┐
│  ESP32S2 (device)           │
│  ─────────────────────      │
│  EmonLib measures V/I/P     │
│  every 15 s → write channel │
│  every 10 s ← poll config   │
└──────────────┬──────────────┘
               │ HTTPS (REST, CORS open)
               ▼
┌─────────────────────────────┐
│  ThingSpeak (cloud)         │
│  ─────────────────────      │
│  Channel 3428306  measures  │
│  Channel 3428310  config    │
└──────────────┬──────────────┘
               │ HTTPS (REST, CORS open)
               ▼
┌─────────────────────────────┐
│  Browser (React website)    │
│  ─────────────────────      │
│  polls every 20 s           │
│  Dashboard / History /      │
│  Settings                   │
└─────────────────────────────┘

## 5.1 The Two Cloud Channels

ThingSpeak stores data in channels, each with up to eight fields. INSTANT uses two channels with a clear division of responsibility:

Measurement channel 3428306 — written by the device, read by the website:

| Field | Meaning | Written when |
| --- | --- | --- |
| field1 | Voltage (V) | every 15 s |
| field2 | Current (A) | every 15 s |
| field3 | Power (W) | every 15 s |
| field4 | Relay state (1 = ON, 0 = OFF) | on change |

Config channel 3428310 — written by the website, read by the device:

| Field | Meaning | Written when |
| --- | --- | --- |
| field1 | vMax (V) | Settings save |
| field2 | iMax (A) | Settings save |
| field3 | pMax (W) | Settings save |
| field4 | Relay command (1 = ON, 0 = OFF) | relay click |

Think of the measurement channel as the device’s diary and the config channel as the control panel that the device checks regularly.

## 5.2 The Free-Tier Constraint That Shapes Everything

ThingSpeak’s free plan allows a channel to be written at most once every 15 seconds. The website respects this by enforcing a 16-second cooldown before any write (a little margin on top of the 15 seconds). Reads are not rate-limited, which is why the browser can poll every 20 seconds without problems. Many of the details in the code exist specifically to handle this 15-second rule gracefully, as you will see in Chapter 19.

## 5.3 Why GitHub Pages + ThingSpeak Works

The magic of this architecture is that everything is public and CORS-open. Because ThingSpeak allows any origin to read and write, the website needs no server. And because GitHub Pages serves static files from a repository, a “deploy” is just copying files to the hosting service. The result is a fully working IoT dashboard with zero servers to rent and zero software to run at night.

# 6. Hardware Components

This chapter describes the physical parts. Even if you never touch the hardware, understanding it helps you read the code comments (which mention pins, sensors, and calibration values).

## 6.1 The Microcontroller — ESP32S2

The ESP32S2 is a small, inexpensive computer on a chip made by Espressif. It has:

- a processor (a CPU that runs programs),

- memory,

- Wi-Fi (so it can reach the internet),

- analog-to-digital converters (ADCs) that turn continuous voltage signals into numbers a program can use.

The firmware is written in C++ using the Arduino framework. The chip reads the sensor pins, does the maths (using a library called EmonLib), writes results to ThingSpeak over HTTPS, and polls ThingSpeak for commands.

Two analog pins are used:

| Pin | Purpose |
| --- | --- |
| 34 | Voltage sensor (ZMPT101B) |
| 35 | Current sensor (SCT-013) |

And one digital pin controls the relay:

| Pin | Purpose |
| --- | --- |
| 18 | Relay (and/or an LED for testing) |

## 6.2 Voltage Sensor — ZMPT101B

The ZMPT101B is a small voltage transformer. It steps the dangerous mains voltage down to a small, safe signal (a few volts) that the ADC can measure. The firmware multiplies the raw reading by a calibration constant (V_CAL) to convert it into real volts. Different power networks need slightly different calibration values, which is why it is a configurable number in the code.

## 6.3 Current Sensor — SCT-013

The SCT-013 is a split-core current transformer — a clamp that clips around a single wire without breaking the circuit. It produces a small signal proportional to the current flowing through the wire. The firmware multiplies the reading by an I_CAL calibration constant to get amps.

Because both sensors are galvanically isolated, the measurement is safe to install and does not interfere with the circuit being measured.

## 6.4 The Relay

A relay is an electrically operated switch. A small current from the microcontroller energizes a coil, which opens or closes a separate, high-power circuit. INSTANT uses the relay to switch a load on or off. The code has a setting called RELAY_ACTIVE_LOW that tells the firmware whether the relay module turns on with a high or a low pin signal — a common gotcha with cheap relay modules (some are “active low”, meaning LOW = ON).

## 6.5 The Measurement Math — EmonLib

The EmonLib library (from OpenEnergyMonitor) implements the classic “Arduino energy monitoring” algorithm:

- Sample the voltage and current signals many times per second (e.g., 20 samples per cycle).

- Compute the RMS voltage and RMS current from those samples.

- Compute real power (the average of voltage × current over time).

- Compute power factor as real power divided by apparent power.

The device feeds these results to ThingSpeak. Note that the website re-derives power factor itself (P/(V·I)) rather than trusting a value sent over the wire, which keeps the cloud contract simple.

# 7. The ThingSpeak Cloud Platform

ThingSpeak is a free IoT analytics service. You create a channel, give it a name and description, and then either your device (or a web page) can write data to it via REST calls. ThingSpeak stores a history of values with timestamps, and lets you fetch the latest value or a window of history.

## 7.1 The Endpoints Used by INSTANT

All requests go to https://api.thingspeak.com. Each channel needs an API key:

- The measurement channel needs a read key so the browser may fetch its data.

- The config channel needs a read key (browser reads it) and a write key (browser writes to it).

| Use | Method & URL | Key |
| --- | --- | --- |
| Latest measurement | GET /channels/3428306/feeds/last.json?api_key=... | read |
| History window | GET /channels/3428306/feeds.json?minutes=N&api_key=... | read |
| Latest config | GET /channels/3428310/feeds/last.json?api_key=... | read |
| Write config/relay | POST /update with field1..field4 and api_key=... | write |

## 7.2 Reading the Latest Value

feeds/last.json returns a JSON object describing the most recent entry, for example:

{
  "created_at": "2026-08-15T10:30:00Z",
  "entry_id": 1234,
  "field1": "230.4",
  "field2": "1.52",
  "field3": "350.2",
  "field4": "0"
}

Notice that ThingSpeak sends field values as text strings, even though they are numbers. The code converts them with parseFloat (see Chapter 14).

## 7.3 Reading History

feeds.json?minutes=N returns up to N minutes of stored history. ThingSpeak downsamples old data, so a 7-day request returns fewer points than a 1-hour request. The browser passes the minutes parameter to choose the range.

## 7.4 Writing

Writing is a POST /update with a form-encoded body containing the channel’s write key plus any of field1–field8. The response is the entry number if the write succeeded, or 0/an error if it was rejected — which is exactly what happens when you try to write twice within 15 seconds.

## 7.5 The 15-Second Rule in Practice

Because the free tier rejects rapid writes, the code treats every write as precious:

- The relay click sets a cooldown timer (16 s) before the next write is allowed.

- The Settings “save” uses the same cooldown.

- If a write is rejected, the UI re-reads the actual stored value instead of guessing.

This rule is arguably the most important operational constraint in the whole project, and it is referenced throughout the code comments.

# 8. How the Website Is Operated (User Guide)

This chapter is the “user manual”. If you open the website, here is what you see and how to use it.

## 8.1 The Header

Across the top there is a bar with:

- The INSTANT logo and name.

- Navigation links: Dashboard, History, Settings.

- A status dot and label showing Live (green) when the newest reading is less than 3 minutes old, or Offline (red) otherwise.

- A theme button (sun/moon) to switch between dark and light mode. The choice is remembered on your device.

## 8.2 The Dashboard

The Dashboard is the home screen. It shows:

- Four metric cards, each with a radial gauge:

  - Voltage (V),

  - Current (A),

  - Power (W),

  - Power Factor (0–1). The numbers animate smoothly when they change (the “gliding” effect), and the gauge fills up as a value approaches its maximum.

- A chart titled “Power consumption — last 5 minutes”, plotting power (left axis) and voltage (right axis) over a rolling five-minute window.

- The relay switch — a large toggle. Clicking it sends an ON/OFF command to the device (more detail in Chapter 18).

- A “Last update” pill showing the time of the newest reading.

- If a safety limit is exceeded, a red FAULT DETECTED banner appears with a Reset button.

## 8.3 The History Page

The History page answers “what happened before now?”:

- Range buttons: 1 hour, 6 hours, 24 hours, 7 days. Selecting one fetches that window from ThingSpeak and redraws the chart.

- A large chart of power and voltage over the selected window.

- Recent readings table — the newest 12 readings with time, V, A, W, and PF.

- Relay activity table — the last 10 relay state changes with timestamps, derived by watching field4 change over time.

## 8.4 The Settings Page

Settings manages the safety thresholds and shows where the app is connected:

- Safety thresholds: voltage max (V), current max (A), power max (W). A badge says “From ThingSpeak” to show the values are coming from the cloud, not from local defaults.

- Relay default state: ON or OFF, which is published along with the thresholds.

- “Publish settings to ThingSpeak” button — writes all four fields to the config channel.

- Android App card — a Download APK button that downloads the INSTANT Android app (see Chapter 22).

## 8.5 Toasts

Small notification pop-ups (bottom-right corner) confirm actions: a green toast for success, red for errors, cyan for informational messages. They disappear automatically after a few seconds and can be dismissed with the x.

## 8.6 What “Live” and “Offline” Really Mean

“Live” does not mean the device is connected — the browser cannot know that. It means: the newest reading in ThingSpeak is fresh (less than 3 minutes old). If the device stopped writing, the reading becomes stale, the status turns Offline, and the dashboard keeps showing the last known data rather than inventing zeros. This honesty is a deliberate design choice.

# 9. The React Basics You Need to Read the Code

The website is written in React, a popular JavaScript library for building user interfaces. If you have never seen React, here are the five ideas you must understand before reading the chapters that follow.

## 9.1 Components

A component is a reusable piece of UI built from JavaScript. In this project components are functions that return a description of what to draw. For example, StatusDot is a function that returns a small colored circle. Components can receive props (short for “properties”) — arguments that customize them. StatusDot on={online} receives a prop named on telling it whether to be green or red.

## 9.2 JSX

JSX is the syntax React uses to write HTML-looking markup inside JavaScript. This snippet:

<span>Live</span>

is actually JavaScript that creates a span element with the text “Live”. JSX lets the code describe the screen next to the logic, which is why this codebase is easy to follow.

## 9.3 State

State is data that changes over time and triggers a redraw when it changes. React components declare state with useState, and the whole application shares some state through a context (see Chapter 13). When state changes, React re-runs the component functions and updates the screen efficiently.

## 9.4 Hooks

Hooks are special functions that let components remember things and react to events:

- useState(initial) — remember a value that can change.

- useEffect(fn, deps) — run code after render (e.g., start a timer, fetch data) and re-run when deps change.

- useRef(initial) — remember a value that survives re-renders without triggering redraws.

- useCallback(fn, deps) — remember a function so it is not recreated every render.

- useContext(...) — read shared state.

The project also defines a custom hook, useAnimatedNumber, which returns a number that smoothly eases toward a target.

## 9.5 Routing

Routing decides which screen is visible. INSTANT uses react-router-dom with a HashRouter: the URL contains a # (for example …/Instant/#/history). The hash makes the app work on static hosting without server-side routing rules.

## 9.6 Styling with styled-components

Instead of separate CSS files, the project uses styled-components: you write CSS inside the JavaScript using template literals, and the library generates real CSS at runtime. A styled component is a component with styles baked in:

const Dot = styled.span`
  width: 10px;
  border-radius: 50%;
  background: ${(p) => (p.$on ? 'green' : 'red')};
`;

Notice the syntax ${(p) => …} — a function that receives the component’s props and returns the CSS value. This is how the theme colors (Chapter 17) are injected everywhere.

# 10. Project Structure and File Map

Here is the layout of the repository. Knowing where everything lives makes the next chapters much easier.

instant/
├── .github/workflows/deploy.yml     # auto-deploys the site to GitHub Pages
├── README.md                        # project documentation
├── docs/INSTANT_Report.md           # this report
├── frontend/                        # the website
│   ├── package.json                 # dependencies and scripts
│   ├── public/
│   │   ├── index.html               # the single HTML page
│   │   ├── manifest.json            # PWA metadata (name, icons)
│   │   ├── favicon.svg              # lightning-bolt logo
│   │   ├── icons/                   # app icons (192px, 512px PNG)
│   │   ├── apk/instant.apk          # the Android app download
│   │   └── .well-known/assetlinks.json  # Android app-identity proof
│   └── src/
│       ├── index.js                 # entry point
│       ├── index.css                # small global CSS
│       ├── App.js                   # routes + theme shell
│       ├── theme.js                 # dark/light palettes + global styles
│       ├── api/client.js            # ThingSpeak REST wrapper
│       ├── context/AppContext.js    # global state + polling engine
│       ├── hooks/useAnimatedNumber.js
│       ├── components/              # Header, StatusDot, MetricCard, Gauge,
│       │                            # AppLineChart, RelaySwitch, FaultBanner,
│       │                            # Toast, Icons
│       └── pages/                   # Dashboard, History, Settings
├── android/twa/                     # Android TWA project (built locally)
└── firmware/INSTANT_ESP32S2/
    └── INSTANT_ESP32S2.ino          # device firmware (legacy MQTT version)

The dependency list in frontend/package.json is small and worth memorizing:

| Package | What it provides |
| --- | --- |
| react / react-dom | The core React library and the DOM renderer |
| react-router-dom | Client-side routing |
| styled-components | CSS-in-JS styling |
| chart.js + react-chartjs-2 | The line charts |
| react-scripts | The build tooling (Create React App) |

# 11. Application Startup — index.js and index.html

## 11.1 The HTML Entry Point

frontend/public/index.html is the only real HTML file. It contains:

- metadata (charset, viewport, theme color, description),

- the app icon links (favicon.svg, the PNG icons, and the PWA manifest.json),

- fonts (Inter for body text, Orbitron for headings),

- a <title>INSTANT · Energy Monitoring</title>,

- an empty <div id="root"></div>,

- a <noscript> warning for users who disable JavaScript.

Everything the user sees is drawn inside #root by JavaScript. The browser never navigates to other pages; routing happens in memory.

## 11.2 The JavaScript Entry Point

frontend/src/index.js is the first JavaScript file the browser runs. It does four things:

import React from 'react';
import ReactDOM from 'react-dom/client';
import { HashRouter } from 'react-router-dom';
import './index.css';
import App from './App';
import { AppProvider } from './context/AppContext';

const root = ReactDOM.createRoot(document.getElementById('root'));
root.render(
  <React.StrictMode>
    <HashRouter>
      <AppProvider>
        <App />
      </AppProvider>
    </HashRouter>
  </React.StrictMode>
);

Reading from the inside out:

- <App /> is the main application.

- <AppProvider> wraps it with the global state context (Chapter 13), so every component can read shared data.

- <HashRouter> enables the #/... routing.

- <React.StrictMode> is a development helper that double-runs effects to catch bugs.

createRoot(...).render(...) is the modern React 18 way to put a component tree into the #root element.

The order of the providers matters: the router must be outside the app so components can use routing, and the context must wrap everything that needs the shared state.

# 12. The App Shell — App.js

App.js is the skeleton of the whole site. It is short, so it is a good first file to study.

export default function App() {
  const { theme } = useApp();

  return (
    <ThemeProvider theme={palettes[theme]}>
      <GlobalStyle />
      <Background />
      <Shell>
        <Header />
        <Main>
          <Routes>
            <Route path="/" element={<Dashboard />} />
            <Route path="/history" element={<History />} />
            <Route path="/settings" element={<Settings />} />
            <Route path="*" element={<Dashboard />} />
          </Routes>
        </Main>
        <Footer>INSTANT · real-time energy monitoring · ThingSpeak</Footer>
      </Shell>
      <ToastHost />
    </ThemeProvider>
  );
}

Let us dissect it:

- ThemeProvider comes from styled-components. It receives the current palette (palettes[theme], either dark or light) and makes it available to every styled component via the theme prop. This is the root of the whole color system.

- GlobalStyle (from theme.js) is a global CSS reset applied once.

- Background is a fixed full-screen layer with animated neon orbs behind the content.

- Shell centers the content with a maximum width.

- Header is the top navigation bar (always visible).

- Routes / Route connect URLs to pages:

  - / → Dashboard,

  - /history → History,

  - /settings → Settings,

  - * (anything else) → Dashboard, a sensible fallback so an unknown URL never shows a blank screen.

- Footer and ToastHost are self-explanatory.

Because every page is a different component rendered in the same shell, the header, background, and footer stay stable while only the middle section changes — that is the “single-page app” feel described in Chapter 4.

# 13. Global State and the Polling Engine — AppContext.js

This is the brain of the website. It holds all the data the pages display, decides when to fetch new data, and exposes the actions (toggle relay, save thresholds, etc.). It uses the Context API: one component (AppProvider) owns the state and provides it to every descendant.

## 13.1 Constants That Govern Behavior

At the top of the file several timing constants control everything:

| Constant | Value | Meaning |
| --- | --- | --- |
| FRESH_MS | 3 minutes | A reading older than this makes the header show Offline |
| POLL_MS | 20 s | How often the browser asks ThingSpeak for data |
| HISTORY_MS | 5 min | Length of the rolling chart window on the Dashboard |
| HISTORY_CAP | 300 | Maximum number of chart points kept in memory |
| WRITE_COOLDOWN_MS | 16 s | Minimum gap between writes (ThingSpeak allows 15 s) |
| RELAY_GRACE_MS | 26 s | After a relay command, ignore config reads that may be stale |

## 13.2 The Initial State

The whole application state starts as one object:

const initialState = {
  voltage: 0,
  current: 0,
  power: 0,
  pf: 0,
  relay: 'OFF',
  fault: null,
  connected: false,
  demoMode: false,
  lastUpdated: null,
  history: [],
  thresholds: { vmax: 240, imax: 15, pmax: 3000 },
  thresholdsSource: 'thingspeak',
  relayLog: [],
  relayCooldownUntil: 0,
};

Every field is displayed somewhere on the site: the four electrical values, the relay state, the fault message, the online flag, the timestamp, the chart history, the thresholds, where the thresholds came from, and the relay cooldown deadline.

## 13.3 Refs for Timely, Bug-Free Updates

Two useRef objects store values that callbacks need without causing re-renders:

- stateRef — always holds the latest state. Because setState is asynchronous, a callback reading state directly could read a stale value; reading stateRef.current is always current.

- lastWriteAtRef — the timestamp of the last write attempt. It is used to enforce the cooldown and to decide whether to trust a config read.

## 13.4 Notifications (notify)

notify(type, message) adds a toast to the list, then schedules its automatic removal after 4.5 seconds:

const notify = useCallback((type, message) => {
  const id = Date.now() + Math.random();
  setToasts((t) => [...t, { id, type, message }]);
  setTimeout(() => setToasts((t) => t.filter((x) => x.id !== id)), 4500);
}, []);

Each toast has a unique id (timestamp plus a random fraction, so two toasts in the same millisecond are still distinct). Toast.js renders the list; dismissToast removes one on demand.

## 13.5 The Polling Loop — the Heartbeat

poll is called immediately on startup and then every POLL_MS (20 s):

const poll = useCallback(async () => {
  let latest = null;
  let config = null;
  try { latest = await getLatest(); } catch (_) {}
  try { config = await getConfig(); } catch (_) {}
  setState((s) => { ... });
}, []);

Two requests run (the measurement channel and the config channel), each guarded so that a failure of one does not break the other. Then a single setState merges everything. Let us look at the merging logic in detail, because it encodes several important rules.

### 13.5.1 Thresholds

const thresholds = config
  ? { vmax: config.vmax, imax: config.imax, pmax: config.pmax }
  : s.thresholds;

If the config read worked, use the cloud thresholds; otherwise keep the previous ones. This is how the Settings page “From ThingSpeak” badge becomes true.

### 13.5.2 Online/Offline

const connected = !!(latest && Date.now() - latest.t < FRESH_MS);

The app is “connected” only if there is a latest reading and it is fresh. This is the honest status logic from Chapter 8.

### 13.5.3 Relay With a Grace Window

const recentlyCommanded = Date.now() - lastWriteAtRef.current < RELAY_GRACE_MS;
const relay = config && !recentlyCommanded ? config.relay : s.relay;

Right after the user sends a relay command, ThingSpeak may still report the old value (the write and the read can race). For 26 seconds the app therefore trusts its own last value instead of the config read, preventing the UI from “snapping back” to the old state.

### 13.5.4 History Update

const point = { t: latest.t, voltage: latest.voltage, current: latest.current, power: latest.power, pf: latest.pf };
const last = s.history.length ? s.history[s.history.length - 1] : null;
const history =
  last && last.t === latest.t
    ? s.history
    : [...s.history, point]
        .filter((p) => p.t > Date.now() - HISTORY_MS)
        .slice(-HISTORY_CAP);

The new reading is appended to the rolling history only if it is different from the last one (deduplication). Old points (older than 5 minutes) are filtered out, and the list is trimmed to 300 points. This keeps the Dashboard chart a tidy, sliding five-minute window.

### 13.5.5 The Fault Check

const exceeded =
  latest.voltage > thresholds.vmax ||
  latest.current > thresholds.imax ||
  latest.power > thresholds.pmax;

If any live reading exceeds its threshold, fault is set to the message 'Safety limit exceeded', which makes the red banner appear on the Dashboard.

## 13.6 toggleRelay — the Cooldown-Protected Write

This is the most subtle function in the app. Read it step by step:

const toggleRelay = useCallback(async () => {
  const now = Date.now();
  const since = now - lastWriteAtRef.current;
  if (since < WRITE_COOLDOWN_MS) {
    notify('info', `ThingSpeak allows one config update per 15 s — retry in ${...} s`);
    return;
  }

  const next = stateRef.current.relay === 'ON' ? 'OFF' : 'ON';
  lastWriteAtRef.current = now;
  setState((s) => ({ ...s, relay: next, relayCooldownUntil: now + WRITE_COOLDOWN_MS }));

  try {
    await saveConfig({ vmax: …, imax: …, pmax: …, relay: next });
    setState((s) => ({ ...s, relay: next }));
    notify('success', `Relay command sent: ${next}`);
  } catch (err) {
    let actual = next;
    try { actual = (await getConfig()).relay; } catch (_) {}
    setState((s) => ({ ...s, relay: actual }));
    notify('error', `${err.message} (1 update / 15 s limit)`);
  }
}, [notify]);

The logic:

- Cooldown check: if less than 16 s since the last write, inform the user and stop. This prevents the free-tier rejection entirely (a “first line of defense”).

- Compute the target: flip the current relay state (ON ↔ OFF).

- Optimistic update: record the write time and immediately flip the UI. The user gets instant feedback; we do not wait for the cloud.

- Write: POST the command (plus the current thresholds, because the write overwrites all fields of the channel) to ThingSpeak.

- Success: keep the optimistic value and show a green toast.

- Failure: never guess. Re-read the config channel to learn the real stored value, show it, and display a red toast. If even the re-read fails, keep the optimistic value.

This function is the direct answer to a real bug: without the cooldown and the grace window, rapid clicks caused the switch to “snap back” to the previous position. Chapter 19 examines the bug story.

## 13.7 saveThresholds — the Settings Writer

saveThresholds(payload) follows the same pattern:

- Enforce the cooldown (returns false so the page can stop its spinner).

- Write all four fields (vMax, iMax, pMax, relay) to the config channel.

- On success, update thresholds and relay in state, set the cooldown, toast success, return true.

- On failure, toast the error and return false.

Note that it writes the relay too. Because each channel update replaces the whole entry, every write must carry all four fields so nothing is accidentally lost. That is why toggleRelay also sends the thresholds and saveThresholds also sends the relay state.

## 13.8 Other Actions

- resetFault() — clears the fault message locally (the banner disappears until the next violating reading arrives).

- toggleTheme() — switches dark/light and saves the choice to localStorage under instant-theme, so the preference survives a page reload.

## 13.9 The Context Value

At the bottom, AppProvider bundles everything into one value object:

const value = {
  state, connected, theme, toasts,
  notify, dismissToast, toggleRelay, saveThresholds, resetFault, toggleTheme,
};
return <AppContext.Provider value={value}>{children}</AppContext.Provider>;

Any component can call useApp() to get this object. useApp also throws a helpful error if it is used outside the provider — a common mistake that would otherwise produce a confusing crash.

# 14. Talking to ThingSpeak — api/client.js

client.js is a small, clean “wrapper” around the ThingSpeak REST API. All the messy URL building, key handling, JSON parsing, and number conversion lives in one place so the rest of the app never deals with HTTP details.

## 14.1 Configuration Object

const TS = {
  base: 'https://api.thingspeak.com',
  channel: '3428306',          // measurements
  readKey: 'UIWSRR7X029RCD5V',
  configChannel: '3428310',    // config
  configReadKey: 'F54BNJ6PACIS3OKD',
  configWriteKey: 'K7TDWWD0WEQN97RR',
};

All channel IDs and keys are constants at the top. This is the single place you would edit to point the app at a different ThingSpeak account.

## 14.2 Number Conversion — num()

function num(value) {
  const n = parseFloat(value);
  return Number.isFinite(n) ? n : 0;
}

ThingSpeak sends numbers as strings; parseFloat converts them. If the value is missing or not a number, the function safely returns 0 instead of NaN (Not-a-Number), which would otherwise poison charts and calculations.

## 14.3 Normalizing a Reading — parseReading()

function parseReading(f) {
  if (!f || f.field1 == null || f.field2 == null) return null;
  const voltage = num(f.field1);
  const current = num(f.field2);
  const power = num(f.field3);
  const relay = String(f.field4) === '1' ? 'ON' : 'OFF';
  const pf = voltage * current > 0 ? Math.max(0, Math.min(1, power / (voltage * current))) : 0;
  return {
    t: Date.parse(f.created_at) || Date.now(),
    voltage, current, power, pf, relay, lastUpdated: f.created_at,
  };
}

This converts one raw ThingSpeak entry into the app’s internal shape:

- Fields are converted to numbers.

- Field4 ("1"/"0") becomes 'ON'/'OFF'.

- Power factor is derived as power / (voltage × current), clamped between 0 and 1, and guarded against division by zero.

- A clean timestamp t (milliseconds) is computed from the created_at text.

If the entry is empty or missing the essential fields, it returns null so callers can skip it.

## 14.4 The Fetch Helper — getJson()

async function getJson(url) {
  const res = await fetch(url, { headers: { Accept: 'application/json' } });
  if (!res.ok) throw new Error(`ThingSpeak error (${res.status})`);
  return res.json();
}

fetch is the browser’s built-in HTTP function. If the server returns an error status, an exception is thrown with a readable message; otherwise the response body is parsed as JSON.

## 14.5 The Four Public Functions

getLatest() — fetches feeds/last.json and normalizes it. This is the “current reading” used by the dashboard.

getHistory(minutes) — fetches feeds.json?minutes=N, maps every entry through parseReading, and drops invalid ones. It also builds a relay log by scanning field4 for changes:

let prev = null;
for (const f of feeds) {
  const st = String(f.field4) === '1' ? 'ON' : 'OFF';
  if (prev !== null && st !== prev) relayLog.push({ t: Date.parse(f.created_at), state: st });
  prev = st;
}

Each time the relay state differs from the previous entry, a change record is created. The History page shows the last 10 of these.

getConfig() — fetches the latest config entry and returns { vmax, imax, pmax, relay }. Note the fallbacks: if a threshold field is empty, it defaults to 240, 15, 3000 — matching the app’s initial state.

saveConfig({ vmax, imax, pmax, relay }) — writes with a POST:

const body = new URLSearchParams({
  api_key: TS.configWriteKey,
  field1: String(vmax), field2: String(imax),
  field3: String(pmax), field4: relay === 'ON' ? '1' : '0',
});
const res = await fetch(`${TS.base}/update`, {
  method: 'POST', body,
  headers: { 'Content-Type': 'application/x-www-form-urlencoded' },
});
if (!res.ok) throw new Error(`ThingSpeak write failed (${res.status})`);

URLSearchParams correctly encodes the body. A non-OK response throws, and the caller (AppContext) handles the error with a toast. This is where the 15-second limit surfaces as an HTTP error.

# 15. Reusable UI Components

The components/ folder contains small, focused pieces. This chapter explains each one and the ideas behind it.

## 15.1 StatusDot

The simplest component: a 10 px circle that is green with a pulse animation when on is true, red otherwise. It proves the pattern props in → styled output.

## 15.2 Header

The navigation bar. Highlights:

- Uses NavLink from react-router-dom so the active link gets a special style automatically.

- Shows the logo in a gradient square, the INSTANT name, three nav links, the live/offline status (StatusDot + text), and the theme toggle.

- const online = connected && state.connected; — both the context value and the state field are used (they are the same, but this shows defensive coding).

## 15.3 MetricCard and Gauge

A MetricCard is one of the four dashboard cards. It receives props: label, value, unit, color, icon, max, and decimals.

Inside, the value passes through the custom hook useAnimatedNumber (see below) so it glides toward the target. The card lays out:

- A header row: colored icon tile + label.

- A gauge box with a Gauge underneath and the number on top of it.

The Gauge component draws two circles with SVG:

- a faint background ring,

- a colored “progress” ring using strokeDasharray and strokeDashoffset.

The math: the full circumference is 2πr. The fraction filled is value/max (clamped to 0–1). The dash offset circumference × (1 − fraction) hides the unfilled part. A CSS transition on stroke-dashoffset makes the ring animate smoothly when the value changes.

const pct = Math.min(1, Math.max(0, value / max));
const offset = circumference * (1 - pct);

## 15.4 useAnimatedNumber — the Gliding Number Hook

A custom hook that smoothly eases a displayed number toward a target over 700 ms using requestAnimationFrame and a cubic ease-out curve:

const tick = (now) => {
  const t = Math.min(1, (now - start) / duration);
  const eased = 1 - Math.pow(1 - t, 3);
  const value = from + (target - from) * eased;
  currentRef.current = value;
  setDisplay(value);
  if (t < 1) raf = requestAnimationFrame(tick);
};

- eased follows the curve 1 − (1 − t)³, which starts fast and finishes gently.

- The current value is stored in a ref so the next animation starts from where the last one ended (no jumps).

- When the target changes, the effect re-runs; when it is done, the animation frame is cancelled to avoid leaks.

This is a great example of a small, reusable piece of polish that makes the dashboard feel alive.

## 15.5 AppLineChart

A wrapper around Chart.js that all charts share. It receives labels (x-axis), series (one or more datasets), a height, and axis titles.

Notable details:

- Registers only the Chart.js modules used, keeping the bundle small.

- Gradient fills: for filled series, a vertical gradient is built from the chart’s drawing area (stronger color at the top, transparent at the bottom).

- Theme awareness: tick and tooltip colors come from useTheme(), so the chart automatically matches dark or light mode.

- Two y-axes: power uses the left axis (y), voltage uses the right axis (y1), because their scales are completely different.

- pointRadius: 0 draws no dots (cleaner for dense data); tension: 0.35 curves the lines.

## 15.6 RelaySwitch

The big glowing toggle. This component’s behavior is tightly coupled to AppContext:

const locked = state.relayCooldownUntil > Date.now();
const handle = async () => {
  if (busy || locked) return;
  setBusy(true);
  await toggleRelay();
  setBusy(false);
};

- busy is local UI state: while a click is in flight, a spinner replaces the icon and clicks are ignored.

- locked is true while the cooldown is active; the button is disabled and shows reduced opacity.

- The knob slides left/right with a springy cubic-bezier transition; the green glow pulses when ON.

- The status line under the switch shows SWITCHING while busy, otherwise RELAY ON or RELAY OFF.

- A useEffect with a 1-second interval forces a re-render every second so the locked state resolves promptly after the cooldown ends.

- Accessibility: role="switch", aria-checked, and an aria-label.

## 15.7 FaultBanner

A red gradient banner with a warning icon, the text FAULT DETECTED, a description, and a Reset button. It is only rendered when fault is truthy (see Dashboard). The Reset button calls resetFault.

## 15.8 Toast

Renders the toast list from AppContext in a fixed container at the bottom-right. Each toast is colored by type (success green, error red, info cyan) via a left border, slides in, and can be dismissed with the x button.

## 15.9 Icons

A tiny icon library of inline SVG components (bolt, wave, zap, pulse, warning, sun, moon, plug). All icons share a common Svg wrapper that sets size, stroke, and stroke width, so icons are just a few path lines each. LogoIcon is simply BoltIcon — the lightning bolt is the brand mark.

# 16. The Pages

## 16.1 Dashboard — pages/Dashboard.js

The Dashboard assembles the components into the home screen. Key logic:

Color coding per metric:

const ACCENTS = {
  voltage: '#00CFFF',
  current: '#FF9F2E',
  power: '#A96BFF',
  pf: '#00E5A0',
};

Chart data construction. The rolling history from state is turned into Chart.js inputs:

const labels = history.map((h) => new Date(h.t).toLocaleTimeString([], { hour12: false }));
const series = [
  { label: 'Power (W)', data: history.map((h) => h.power), color: ACCENTS.power, fill: true, borderWidth: 2 },
  { label: 'Voltage (V)', data: history.map((h) => h.voltage), color: ACCENTS.voltage, fill: false, borderWidth: 1.5, yAxisID: 'y1' },
];

Gauge ranges. Each MetricCard picks a sensible maximum so the gauge is meaningful:

- Voltage: Math.max(260, thresholds.vmax * 1.1) — a bit above the safety limit.

- Current: Math.max(15, thresholds.imax * 1.5).

- Power: Math.max(2500, thresholds.pmax).

- Power factor: always 1.

Layout. Four cards in a responsive grid; below them, the chart (2/3 width) and the relay switch (1/3 width). The fault banner appears between the header row and the cards when present.

## 16.2 History — pages/History.js

The History page manages its own state (it does not need global state):

const [range, setRange] = useState(1440);
const [data, setData] = useState(null);
const [loading, setLoading] = useState(false);

- range — the selected window in minutes (defaults to 24 hours = 1440).

- data — the fetched { points, relayLog }.

- loading — whether a fetch is in flight.

The fetch effect is the interesting part:

useEffect(() => {
  let active = true;
  setLoading(true);
  getHistory(range)
    .then((res) => { if (active) setData(res); })
    .catch((err) => { if (active) notify('error', err.message || 'Failed to load history'); })
    .finally(() => { if (active) setLoading(false); });
  return () => { active = false; };
}, [range, notify]);

- active is a flag flipped to false when the component unmounts or the range changes. This prevents a slow, outdated response from overwriting a newer one (the classic race condition guard).

- The effect re-runs whenever range changes, so clicking “7 days” automatically fetches the new window.

- Failures become error toasts rather than blank screens.

Data shaping. The fetched points become chart labels (full date + time for history) and two series. The recent readings table shows the last 12 points reversed (newest first). The relay activity table shows the last 10 relay log entries, with colored ON/OFF badges.

Empty states. When there is no data yet, the page says so (“No history yet — waiting for data from ThingSpeak.”) instead of drawing an empty chart — a nice touch for beginners to notice.

## 16.3 Settings — pages/Settings.js

The Settings page is a form. Key ideas:

Form state. Three text inputs (vMax, iMax, pMax) plus a relay selector, all as useState strings. A useEffect keeps the form synced to the loaded state:

useEffect(() => {
  const th = state.thresholds;
  if (th.vmax) {
    setVmax(String(th.vmax));
    setImax(String(th.imax));
    setPmax(String(th.pmax));
  }
  setRelayDefault(state.relay);
}, [state.thresholds, state.relay]);

Validation before writing. validate() checks the numeric ranges (voltage 1–400, current 0.1–100, power 1–100000) and returns an error message if invalid. handleSave shows the error as a toast and refuses to write — bad values never reach the cloud.

The save. saveThresholds from context does the write and returns true/false; the button shows “Publishing…” while waiting.

Info panels. The Note texts are mini-documentation: which channel fields each value maps to, the REST endpoint, and the polling cadence. The Android App card (with the Download APK button) is described in Chapter 22.

# 17. Theming and Styling

## 17.1 Two Palettes

theme.js defines a palettes object with a dark and a light palette. Each palette is a set of named colors:

| Key | Dark value | Light value | Used for |
| --- | --- | --- | --- |
| bg | #0B0D17 | #F1F3FA | page background |
| bg2 | #141830 | #FFFFFF | raised surfaces / tooltips |
| text | #E9ECFF | #151A33 | main text |
| textDim | #8A91B4 | #5A6288 | secondary text |
| cyan | #00D2FF | #00A2C9 | brand / links |
| magenta | #FF007F | #E60077 | brand accent |
| green | #00E5A0 | #00B080 | success / relay ON |
| red | #FF3B6B | #E02D55 | errors / faults |
| amber | #FFD166 | #C99A1E | warnings |
| … | … | … | … |

Because every styled component reads colors through (p) => p.theme.color, switching the palette re-colors the entire site in one place.

## 17.2 Global Styles

GlobalStyle (via createGlobalStyle) applies CSS that is not component-specific:

- box-sizing: border-box for predictable sizing,

- font families (Inter for text, Orbitron for headings),

- the background/color with a smooth transition,

- keyframe animations slideIn and pulseDot used by toasts and status dots.

## 17.3 The Animated Background

Background is a fixed, full-screen div behind everything. It uses two radial gradients (magenta top-right, cyan bottom-left) plus two blurred, drifting “orbs” animated with a slow drift keyframe. All the visual drama of the site comes from CSS — no images required.

## 17.4 The Global CSS File

index.css is deliberately tiny: a color-scheme: dark hint, a body margin reset, and styled scrollbars. The comment says it plainly: “the real theming lives in styled-components GlobalStyle.”

# 18. End-to-End Feature Walkthroughs

This chapter traces real user actions through the entire stack, tying together every chapter before it.

## 18.1 “Turn the relay OFF”

What the user does: On the Dashboard, clicks the big relay toggle.

What happens, step by step:

- RelaySwitch’s handle() runs. busy becomes true (spinner shows).

- toggleRelay() in AppContext checks the cooldown. If less than 16 s since the last write, it shows “retry in N s” and stops.

- The cooldown is passed; the UI optimistically flips to OFF (setState({ relay: 'OFF' })) and records relayCooldownUntil.

- saveConfig({ vmax, imax, pmax, relay: 'OFF' }) in client.js builds a form body and POSTs to api.thingspeak.com/update with the config channel’s write key.

- ThingSpeak stores the entry. The device’s next 10-second poll reads field4 = 0.

- The device firmware applies the command, switches the relay off, and writes its actual state to the measurement channel (field4 = 0).

- Meanwhile the browser continues polling every 20 s. For 26 s after the write, the poll ignores the config read for the relay (grace window) so the OFF state is not overwritten by a stale read.

- After the grace window, polls trust the config read again. The next successful read confirms OFF.

- A green toast confirms: “Relay command sent: OFF”.

Delays the user can expect: the toggle flips instantly (optimistic), the physical relay switches within ~10 s (the device’s poll interval), and the cloud-confirmed state appears within ~20 s.

## 18.2 “Save new thresholds”

- On Settings, the user edits vMax/iMax/pMax and clicks “Publish settings to ThingSpeak”.

- validate() checks the ranges; if invalid, an error toast appears and nothing is sent.

- saveThresholds enforces the same 16 s cooldown as the relay.

- saveConfig writes all four fields (including the relay, so nothing is lost).

- On success, state thresholds update instantly, the Settings inputs re-sync, a success toast appears, and the fault banner (if any) now compares readings against the new limits on the next poll.

## 18.3 “Check what happened yesterday”

- The user opens History and clicks “24 hours”.

- setRange(1440) triggers the effect; getHistory(1440) GETs feeds.json?minutes=1440.

- client.js maps every entry into a point and derives the relay log from field4 changes.

- The page redraws the chart, the recent-readings table (last 12, newest first), and the relay-activity table (last 10 changes).

- Because the data lives in ThingSpeak, the history is available even when the device is offline — it is a stored record, not a live stream.

## 18.4 “Why does the status say Offline?”

The header computes connected = latest exists AND fresh (< 3 min). Offline appears when:

- ThingSpeak is unreachable from the browser, or

- the latest stored reading is older than 3 minutes (device powered off, Wi-Fi lost, or it stopped writing).

The site keeps showing the last known numbers (it does not zero them), and the status dot turns red. Once a fresh reading arrives, everything recovers automatically — no reload needed.

# 19. Error Handling and Edge Cases

A good dashboard is judged by how it behaves when things go wrong. This chapter catalogs the failure modes INSTANT handles and how.

## 19.1 The Rate-Limit Bug That Shaped the Design

In an earlier version of the app, clicking the relay twice quickly caused a strange bug: the switch flipped, then snapped back to the previous position, and a second click was needed. The cause was the ThingSpeak free-tier rule — one write per 15 seconds per channel.

The first write succeeded; the second was rejected with HTTP 400, and the old code reacted to the failure by reverting to the (stale) config value. The fix had three parts, all visible in the code:

- WRITE_COOLDOWN_MS = 16000 — refuse to write twice within 16 s, so the rejection is never triggered in the first place.

- RELAY_GRACE_MS = 26000 — after a command, ignore config reads for 26 s so an in-flight stale read cannot overwrite the optimistic UI.

- No blind reverts — on a write failure, the app re-reads the actual stored value and shows the truth, rather than guessing.

This is a textbook example of designing software around a real-world constraint, and it is why the cooldown logic appears in both toggleRelay and saveThresholds.

## 19.2 ThingSpeak Unreachable

getLatest/getConfig throw on network errors. poll catches each request separately:

- If the latest-read fails, connected becomes false and the dashboard keeps the previous values.

- If the config-read fails, the previous thresholds are kept.

The two are independent, so one failure does not blank the whole dashboard.

## 19.3 Bad or Missing Data

- Missing fields → num() returns 0 instead of NaN.

- Entries missing essential fields → parseReading returns null, and callers filter those out.

- Power factor division by zero → guarded (voltage * current > 0), clamped to 0–1.

## 19.4 Empty History

History shows friendly “No history yet” messages instead of an empty chart, and the Dashboard handles a zero-length history gracefully (empty labels/series).

## 19.5 Form Validation

Settings refuses to publish out-of-range thresholds, so garbage never reaches the cloud or the device.

## 19.6 Race Conditions

- The History page’s active flag prevents stale responses from overwriting newer ones when the user switches ranges quickly.

- stateRef ensures async callbacks always read the current state.

- The relay grace window prevents read/write races after a command.

## 19.7 Unknown URLs

<Route path="*" element={<Dashboard />} /> means an unknown URL shows the Dashboard rather than a blank page or a confusing 404.

# 20. Security Considerations

It is important to be honest about the security model of a project like this.

## 20.1 Keys in the Browser

The ThingSpeak read and write keys are embedded in the JavaScript bundle. Anyone can open the site, view the source, and read (or write) the channels. This is by design for a public demo, but it is a genuine limitation:

- Anyone can change the thresholds or toggle the relay.

- This is acceptable only because the project is a demonstration.

## 20.2 How a Real Deployment Would Differ

For production you would:

- Use private ThingSpeak channels.

- Keep write keys server-side only — a small backend would receive browser requests, authenticate users, and perform the writes itself, so keys never reach the browser.

- Add authentication (accounts) so only authorized people can control the relay.

- Note: even with a private channel, reads from the browser still expose the read key — the write key is the one that really matters for control.

## 20.3 Input Validation

All threshold inputs are validated in the UI before publishing, reducing the chance of nonsensical values reaching the device (though a malicious user could bypass the UI — see 20.1).

## 20.4 Physical Safety

The firmware is designed to cut the relay when readings exceed thresholds, as a last line of defense independent of the website. Web-based control is best treated as a convenience, not a safety mechanism.

# 21. Building, Running, and Deploying

## 21.1 Running Locally (Development)

With Node.js installed:

cd frontend
npm install        # first time only
npm start          # starts the dev server on http://localhost:3000

The dev server gives you hot reloading (edit a file, see the change instantly) and helpful error overlays. No environment files or keys to configure — the keys are in the source.

## 21.2 Building for Production

npm run build

Create React App compiles the JSX/CSS into optimized static files in frontend/build:

- HTML, CSS, and a bundle of JavaScript with hashed filenames (e.g. main.c627c1e2.js). The hash changes whenever content changes, which helps browsers cache correctly.

The build is fully portable: because the site talks to ThingSpeak directly (CORS open) and uses HashRouter plus "homepage": "." (relative paths), you can serve frontend/build from any static host — or even open index.html from disk.

## 21.3 Automatic Deployment to GitHub Pages

.github/workflows/deploy.yml is a GitHub Actions workflow that runs on every push to main:

- Checks out the repository.

- Installs Node 22.

- Runs npm ci in frontend (installs exact versions from the lockfile).

- Runs npm run build.

- Uploads frontend/build as a Pages artifact.

- Deploys it to GitHub Pages via actions/deploy-pages.

The result: pushing code to GitHub automatically publishes the new site at https://vwoudka.github.io/Instant/ within about a minute. This is called continuous deployment — no manual steps.

## 21.4 Why the Workflow Only Builds the Frontend

The workflow builds only frontend. The device firmware is uploaded to the ESP32 with the Arduino IDE (it is not deployed through the website pipeline), and the Android app is built locally (Chapter 22). The GitHub workflow is purely the website’s release pipeline.

# 22. The Android App (APK)

The website can also be installed as an Android app. It is a Trusted Web Activity (TWA): a thin Android shell that opens the website full-screen in Chrome, showing an app icon in the launcher and behaving like a native app — but it is just the website in a window.

## 22.1 The Pieces

Three things make this work:

- A web app manifest (frontend/public/manifest.json) with the app name, start_url, display mode, theme color, and icons (192 px and 512 px PNG). This makes the site installable as a PWA.

- The APK itself (frontend/public/apk/instant.apk) — a ~1.2 MB signed Android package built with the Bubblewrap tool, whose manifest declares the package name app.instant.energy and the URL it opens.

- assetlinks.json (frontend/public/.well-known/assetlinks.json) — a JSON file proving that the website legitimately owns the app. It lists the package name and the SHA-256 fingerprint of the app’s signing certificate. Android checks this file when verifying the TWA, which is why it must live at a fixed path on the site.

## 22.2 How a User Gets It

On the Settings page, the Download APK button links to ./apk/instant.apk (with a download attribute). The user downloads it to their phone, allows installing apps from unknown sources, and installs it. Because the APK is hosted by the website itself, new versions are simply a new APK at the same URL — the site is the app store.

## 22.3 Rebuilding the APK

The Android project lives in android/twa and is git-ignored — it is a local build toolchain (JDK 17 + Android SDK + Bubblewrap), not part of the website repository. To rebuild:

- Ensure the JDK, Android SDK, and @bubblewrap/cli are installed (they were set up during the initial build).

- Re-run the TWA generator and bubblewrap build from android/twa.

- Copy app-release-signed.apk to frontend/public/apk/instant.apk.

- Commit and push; the workflow redeploys the site with the new APK.

The signing keystore and its password are kept locally (not in git), so the app can be updated with the same identity.

# 23. The ESP32 Firmware

The repository contains the device program: firmware/INSTANT_ESP32S2/INSTANT_ESP32S2.ino.

Important note for readers: the firmware file currently checked into the repository is a legacy MQTT version from an earlier architecture. The current deployed system uses the ThingSpeak architecture described throughout this report (the device writes the measurement channel and polls the config channel), and the README describes that ThingSpeak firmware. The MQTT sketch is kept for reference. The hardware theory below applies to both versions.

## 23.1 What the Firmware Does (ThingSpeak Architecture)

A ThingSpeak-based sketch would:

- Connect to Wi-Fi.

- Every 15 s (T_SEND): measure V/I/P with EmonLib and write field1..field3 (plus relay state in field4) to the measurement channel.

- Every 10 s (T_READ): poll the config channel, read field1..field4, and apply new thresholds or a new relay command.

- Enforce safety: if a reading exceeds vMax/iMax/pMax, force the relay off and report a fault.

The README documents the wiring table:

| Item | Value |
| --- | --- |
| Voltage sensor | ZMPT101B on pin 34 |
| Current sensor | SCT-013 on pin 35 |
| Relay / LED | pin 18 |
| Calibration | V_CAL, I_CAL, PHASE |
| Relay polarity | RELAY_ACTIVE_LOW |
| Write cadence | 15 s (T_SEND) |
| Config poll | 10 s (T_READ) |

## 23.2 Reading the Legacy MQTT Sketch

Even though it is not the current transport, the checked-in sketch teaches the hardware fundamentals:

- Setup: configures the relay pin to a safe state, sets ADC resolution/attenuation, configures EmonLib for the voltage and current pins, calibrates with a few empty readings, connects to Wi-Fi, and connects to the MQTT broker.

- The loop is time-sliced with millis() (no delay() blocking):

  - every 5 s: reconnect Wi-Fi/MQTT if needed,

  - every 1 s: take a measurement,

  - every 2 s: publish measurements,

  - continuously: mqtt.loop() keeps the connection alive, and controlRelay() applies safety logic.

- Safety logic (controlRelay): trips the relay if voltage is too high, too low (undervoltage), current too high, or power too high. After a trip, a 5-second “cool-down” must pass before a new command is accepted.

- MQTT message handling: subscribes to relay commands and threshold topics, updating local variables when messages arrive.

The MQTT version publishes every 2 s (MQTT has no rate limit), whereas ThingSpeak’s 15 s write limit forces the slower cadence — a perfect illustration of how the cloud platform shapes the device behavior.

# 24. Troubleshooting

A practical table of symptoms and fixes, drawn from the project’s own README and from the way the code is written.

| Symptom | Likely cause | What to do |
| --- | --- | --- |
| Blank / white page | Opening public/index.html directly (it has no compiled JS) | Open the built frontend/build/index.html, or serve the build folder |
| Header shows Offline | Device stopped writing, or ThingSpeak unreachable | Check the ESP is powered and Wi-Fi credentials are correct; the site will recover automatically when a fresh reading arrives |
| Gauges stay at 0 | No fresh feeds in the measurement channel, or bad sensor wiring/calibration | Check the channel has recent entries; verify sensor wiring and V_CAL/I_CAL |
| Relay toggle has no effect | Config write failed (check the toast), or the device is not running | Retry after the 16 s cooldown; confirm the firmware is polling the config channel |
| Relay switch seems to “snap back” | (Old behavior — now fixed) writes within 15 s are rejected | Update the site; if it still happens, watch for error toasts and wait for the cooldown |
| Chart is empty | Selected range has no stored feeds yet | Pick a smaller range (e.g. 1 hour) |
| “Retry in N s” toast on relay/Settings | The 16 s write cooldown is active | Wait a few seconds and try again |
| APK won’t install | “Unknown sources” is disabled | Allow installing apps from unknown sources; verify you downloaded the whole file |
| APK opens but shows a message instead of the site | assetlinks verification failing | Confirm .well-known/assetlinks.json is deployed and matches the package + fingerprint |

# 25. Glossary

| Term | Definition |
| --- | --- |
| API | Application Programming Interface — a set of defined requests a service accepts (e.g. “give me the latest reading”). |
| API key | A secret string used to authenticate a request to a service. |
| Apparent power | V × I — the power the supply must provide, ignoring whether it is used usefully. |
| CORS | Cross-Origin Resource Sharing — the mechanism by which a server allows browsers from other origins to read it. |
| Channel | In ThingSpeak, a container of up to 8 fields of data with timestamps. |
| Component | A reusable piece of UI in React, written as a function returning JSX. |
| Context | A React feature for sharing state across many components without passing it through every level. |
| Cooldown | A minimum delay before an action may be repeated (here, 16 s before the next write). |
| Current (I) | The flow of electric charge, measured in amperes (A). |
| Deploy | Publishing a built version of the software to the hosting service. |
| fetch | The browser’s built-in function for making HTTP requests. |
| Field | In ThingSpeak, one named column of a channel’s data. |
| Frontend | The part of an application that runs in the browser. |
| GitHub Actions | GitHub’s automated workflow system; used here to deploy the site on every push. |
| GitHub Pages | GitHub’s free static website hosting. |
| Hook | A React function (useState, useEffect, …) that adds features to a component. |
| HTTP | The protocol browsers and devices use to send requests and receive responses. |
| IoT | Internet of Things — physical devices connected to the Internet. |
| JSON | JavaScript Object Notation — a text format for structured data. |
| JSX | The HTML-like syntax used inside React JavaScript. |
| Polling | Repeatedly asking a server “is there anything new?” on a timer. |
| Power (P) | The rate of using energy, in watts (W); P = V × I. |
| Power factor (PF) | Real power ÷ apparent power; a value from 0 to 1. |
| Prop | An argument passed to a React component. |
| React | A JavaScript library for building user interfaces from components. |
| Real power | The power actually doing useful work, measured by averaging V×I. |
| Relay | An electrically operated switch. |
| REST | A style of HTTP API organized around resources and GET/POST verbs. |
| RMS | Root-mean-square — the standard measure of an AC voltage or current. |
| Single-page app (SPA) | A website that loads once and swaps content in the browser. |
| State | Data a component remembers; changes to it re-render the UI. |
| styled-components | A library for writing CSS inside React components. |
| TWA | Trusted Web Activity — an Android app shell that shows a website full-screen. |
| Threshold | A safety limit (vMax, iMax, pMax) that must not be exceeded. |
| Toast | A small transient notification pop-up. |
| Voltage (V) | Electrical “pressure”, measured in volts (V). |

# 26. Further Reading

If this report made you curious, here are good next steps, roughly in order of difficulty.

For the web side: - The React official tutorial: https://react.dev/learn - styled-components documentation: https://styled-components.com/docs - Chart.js documentation: https://www.chartjs.org/docs/ - How HTTP works (MDN): https://developer.mozilla.org/docs/Web/HTTP

For the IoT/cloud side: - ThingSpeak documentation: https://docs.thingspeak.com - ThingSpeak REST API reference: https://www.mathworks.com/help/thingspeak/rest-api.html

For the hardware side: - OpenEnergyMonitor’s “How to build an Arduino energy monitor”: https://docs.openenergymonitor.org - ESP32 Arduino core: https://github.com/espressif/arduino-esp32

In this repository: - README.md — concise project documentation including the API reference table. - The source files under frontend/src — now that you have read this report, the code should read like a story: client.js (how data comes in), AppContext.js (how it is kept fresh and written back), the components (how it is shown), and the pages (how it is assembled).

End of report. INSTANT — real-time energy monitoring, from sensor to screen.